How to install PARTHENON system (Ver. 2.3.0.6 for SparcStation)

1. Put "par2306.tgz" in any directory (/xxxx) where you can access.

2. Extract PARTHENON system.

% tar xvzf par2306.tgz   (in case of GNU tar)

Then, sub-directories are created as follows:

par2306/cell_lib.dir  : sample cell libraries

par2306/com           : executables (with the licence file "parthe.non")

par2306/doc.dir       : documents (in Japanese)

par2306/example.dir   : design examples

par2306/sfl_lib.dir   : circuit libraries (synthesized netlist)

par2306/sfl_lib.src   : circuit libraries (SFL source)

You may change the directory name "par2306" to any name you like.

3. Set an environment variable "PARTHENON".

% setenv PARTHENON /xxxx/par2306

Add "/xxxx/par2306/com" to your search path.

You can also modify your ".login" or ".cshrc" to fit your system.

4. Confirm the installation of your PARTHENON system.

% cd $PARTHENON/example.dir

% seconds

SECONDS> cpu.scr

Then, simulation for "cpu.sfl" will run. You can exit by "bye" command.

SECONDS> bye

You can use a command "auto" as an easy way to synthesize SFL descriptions.

To make a postscript file of circuit diagram from "cpu.sfl", just type:

% auto cpu ps DEMO demo

In order to use "auto" command, the name of SFL file "NAME.sfl"  must  be

the same as its top module name "NAME".

Read "$PARTHENON/com/Makefile" to understand how "auto" works.

More information:

http://www.kecl.ntt.co.jp/car/parthe/